Our Ref:doc/gen/epc/dec

To:The Town Mayor and Members of the Council

Meeting 21st June 2023

Energy Performance Certification (EPC)

Display Energy Certification (DEC)

Energy Efficiency Recommendations.

PUPOSE OF THE REPORT

To consider the circulated information in relation to a recent report received from Mr. M. Ellis, Surveyor, Carbon Certification, to consider the background information, recommendations and financial implications contained within this report.

BACKGROUND

To ensure Whiston Town Council are fully compliant with the requirements of energy performance certification and display energy certification at each of it’s three sites it has been necessary to instruct an accredited professional to carry out the required assessment and calculations.

Whiston Town Hall

It is important that Members are aware that this building was built in 1973 and building regulations Part L were implemented in 1976 therefore energy efficiency was not measured at the time.

This building currently has in place an EPC which will expire in April 2024, it is not advisable to lodge a renewal before this time.  The findings of the recent inspection / calculation will not be lodged until April 2024.  On initial findings this building will fall below the MEES (Minimum Energy Efficiency Standard) as a category of band F has been identified, which is due in the main to the lack of insulation in walls and ceilings and therefore it is recommended that insulation work be carried out before April 2024 if Members wished to ensure a rating of band E.

The heating boiler at this site is 81% efficient and a replacement may be considered in the long term.  Due to the age of this building and the significant area of flat roofing it is not likely to improve to above a D rating without significant investment in providing pitched roofing etc.

This building now displays a DEC in line with the legal requirements. The implementation of this will ensure that the annual energy usage is monitored and recorded.

George Howard Centre

This building is currently rated as band D and again would benefit from Insulation to the walls and roof space area along with photovoltaics.

The heating boiler efficiency at this site is 85%

This building also displays a DEC in line with the legal requirements. The implementation of this will ensure that the annual energy usage is monitored and recorded.

Robert Foulkes Centre

Again this building is currently rated as band D and would also benefit from Insulation to the walls and roof space area along with photovoltaics.

The heating boiler efficiency at this site is 85%

There is no legal requirement for a DEC at this site.

Members will note that each building does currently comply with the MEES rating. Although improvements could be made, it is recommended that the fabrication of the building should always be considered first. It is hoped that the central government will allocate funding under the PAS 2038 program, going forward.

The Assessors recommendation prior to April 2024 is to consider insultation to the fabrication of this building to avoid being categorized below the Measured Energy Efficiency Standard.